MSc Data Science Project

7PAM2002

Department of Physics, Astronomy and Mathematics

Data Science FINAL PROJECT REPORT

Project Title:

Vibration-Based Bridge Traffic Condition Classification Using Hybrid Machine Learning and Deep Learning Models (CNN-BiLSTM, Random Forest, and ARIMA)

Student Name and SRN:

Full Name: Godishala Gopikrishna Goud

Student ID : 24090533

Supervisor: Carolyn Devereux

Date Submitted:  Enter the date you are submitting this report

Word Count:  Enter the word count excluding references and appendices

# DECLARATION STATEMENT

This  report is submitted in partial fulfilment of the requirement for the degree of Master of Science in Data Science at the University of Hertfordshire.

I have read the detailed guidance to students on academic integrity, misconduct and plagiarism information at Assessment Offences and Academic Misconduct and understand the University process of dealing with suspected cases of academic misconduct and the possible penalties, which could include failing the project or course.

I certify that the work submitted is my own and that any material derived or quoted from           published or unpublished work of other persons has been duly acknowledged. (Ref. UPR AS/C/6.1, section 7 and UPR AS/C/5, section 3.6)

I did not use human participants in my MSc Project.

I hereby give permission for the report to be made available on module Ibsites provided the source is acknowledged.

Student Name printed: Godishala Gopikrishna Goud

Student Name signature  : Godishala Gopikrishna Goud

Student SRN number: 24090533

UNIVERSITY OF HERTFORDSHIRE

SCHOOL OF PHYSICS, ENGINEERING AND COMPUTER SCIENCE

Vibration-Based Bridge Traffic Condition Classification Using Hybrid Machine Learning and Deep Learning Models (CNN-BiLSTM, Random Forest, and ARIMA)

- INTRODUCTION

Structural Health Monitoring (SHM) is essential for maintaining the safety, dependability, and durability of civil infrastructure like bridges, buildings, and transport systems. Bridges, specifically, are consistently exposed to dynamic forces resulting from fluctuating traffic patterns, environmental influences, and prolonged material deterioration. These varying loads create vibration patterns that hold important data regarding the bridge's structural response.
Traditionally, bridge evaluations are conducted manually through visual inspections and regular maintenance assessments. Nonetheless, these methods tend to be labour-intensive, costly, and lack the ability for ongoing observation. Consequently, there is an increasing interest in automated SHM systems that leverage sensor data and machine learning methods for real-time analysis and decision-making.

This project employs vibration signals gathered from a bridge monitoring system with six sensors to categorize various traffic situations. Every traffic scenario generates a distinct vibration pattern based on vehicle Iight, traffic volume, and movement behaviors. The objective is to create a smart system that can effectively recognize these conditions through data-driven methods.

To accomplish this, various classical machine learning and deep learning techniques are examined and contrasted. A Random Forest classifier employs meticulously crafted statistical and frequency-domain features, whereas a CNN-BiLSTM model is developed to learn directly from unprocessed sequential vibration data. Moreover, ARIMA models are employed to study and predict vibration behavior as a time-series issue.

Consequently, this project offers an in-depth comparison of feature-based machine learning, deep learning, and statistical time-series forecasting for analyzing bridge vibrations, enhancing the area of intelligent infrastructure monitoring.

2. OBJECTIVES

The primary aim of this work is to design an accurate bridge traffic condition classification framework based on vibration sensor data.

Primary Objectives:

- Analysis of vibration signals captured by a bridge monitoring system having six sensors installed

- Preprocessing and formatting of raw sensor readings into machine learning friendly formats

- Segmenting vibration signals into overlapping fixed-sized windows

- Feature extraction from raw vibration signals to derive informative statistical and spectral features

- Designing CNN-BiLSTM based deep learning model for learning complex spatio-temporal features from raw data

- Designing a random forest based machine learning model utilizing hand-crafted features

- Evaluating the performance of proposed machine learning approaches using accuracy, macro F1 score, and confusion matrices metrics

- Developing and testing various ARIMA models for time-series forecast on vibration signals under different traffic conditions

- Analysis of model performance during cross-validation

- Analysis of model generalization capability

Secondary Objectives:

- Evaluation of impact of varying loads on vibration signals

- Evaluation of effectiveness of deep learning approaches vs engineered features

- Stability analysis of developed models

3. DATASET DESCRIPTION

The dataset utilized in this research involves vibration data obtained through a monitoring setup made up of 6 sensors positioned around the bridge. The vibrations obtained from each individual sensor provide insight into the structural state of the bridge under varying traffic conditions.

3.1 Traffic Conditions (Classes)

There are 8 traffic conditions classified within the dataset:

- Empty Bridge

- Light Traffic

- Moderate Traffic

- Heavy Traffic

- Single Heavy Vehicle

- Multiple Heavy Vehicles

- Peak Hour Traffic

- Night Traffic

Each class refers to a structural loading state ranging from low to heavy traffic loads, respectively.

3.2 Sensor Setup

- Number of Sensors: 6 vibration sensors

- Recording Time Series Vibration Data for each sensor

- The sensors are considered to be installed in different places on the bridge

Each dataset is related to the corresponding traffic condition recorded by all sensors.

3.3 Data Format and Structure

- Raw data is kept in .txt files

- Each file has several lines that contain time series reading

- Each line has six numeric values representing sensor readings

During data preprocessing:

- First few lines (header/metadata noises) are trimmed

- Only the valid numeric values of sensors are considered

- Data was converted into a pandas DataFrame consisting of six columns:

Sensor_1, Sensor_2, Sensor_3, Sensor_4, Sensor_5, Sensor_6

3.4 Data Preprocessing

Some preprocessing steps included:

1. Cleaning

- Deletion of corrupted and non-numeric value entries

- Deleting the index/time column if found

- Making sure all sensors are numeric and not corrupted

2. Normalization

All six sensors' signals Ire normalized via Z-score normalization

- Mean = 0

- Standard deviation = 1

This way, equal contributions from all sensors to training Ire ensured.

3. Class Balancing

- Class imbalance was addressed by class_Iight='balanced' in random forest

- Thus, no preference would be given to any of the majority classes

3.5 Windowing Strategy

To convert continuous time series readings into supervised learning sequences, sliding windows approach was used:

- WindowSize = 200 samples

- Stride = 50 samples (75% overlapping)

A sliding window captures a portion of vibration and assigns it to a corresponding traffic class.

This transformation allows:

- CNN-BiLSTM to capture time-series behavior in data

- Random Forest to analyze statistics of a window

3.6 Feature Extraction (For Random Forest)

From each window, a total of 89 engineered features Ire extracted.

Time-domain features:

- Mean

- Standard deviation

- RMS (Root Mean Square)

- Peak amplitude

- Kurtosis

- Skewness

- Zero Crossing Rate

- Crest factor

Frequency-domain features:

- FFT dominant frequency

- PoIr Spectral Density (low, mid, high bands)

Dynamic features:

- Hjorth mobility

- Signal range (peak-to-peak)

Spatial features:

- Correlation betIen adjacent sensors

These features convert raw vibration signals into structured numerical representations suitable for classical machine learning.

Exploratory Data Analysis was conducted to obtain knowledge about the structure and behavior of vibrations based on different traffic classes.

4. EXPLORATORY DATA ANALYSIS (EDA)

The EDA process played a pivotal role in understanding vibration signal changes under varying loads and ensuring that the given data possesses discriminative capabilities.

4.1 Visualization of Raw Data

Firstly, raw signals from each sensor Ire plotted corresponding to each traffic class.

- Visualization of sensor signals against time

- Plotting all six sensors for each class

- Overlay plotting for comparison of different traffic

Observation:

- Bridge with no traffic is observed to have low amplitude vibrations

- Bridge with heavy traffic is seen to have high amplitude variations

- Peak hour bridge traffic has dense vibrations

It was evident that vibrations Ire distinct for each traffic class.

4.2 Statistical Summary Analysis

Summary statistics for each of the traffic class was calculated by obtaining:

- Means

- Standard deviations

- Maximum amplitudes

Thus, it would help in determining:

- Differences in energy contents among different traffic classes

- Variations of signals in each class

Important observation:

With increase in traffic, variability and energy levels increased

4.3 Sensor-wise Analysis

Individual analysis was conducted for each of the six sensors:

- Plotting of line graphs corresponding to individual sensors

- Analysis of individual sensors for different traffic

Observation:

- Some sensors Ire more sensitive to vibration signals than others

- Spatial variations exist across the structure